Robot Greeter — Project Charter & Requirements (v0.2)

Status: Draft for review | Owner: Stephen Perelgut | Date: 2025-10-15

1. Executive Summary

Design and deploy an indoor, human‑friendly mobile robot that patrols a defined space, detects visitors, greets them, and—if consented and enrolled—addresses known individuals by name. Unknown visitors receive a privacy‑respectful greeting and can be handed off to a human operator via a companion app over Bluetooth Low Energy (BLE). The MVP emphasizes reliability, on‑device perception (for privacy and latency), simple enrollment with consent, and safe autonomous navigation.

2. Objectives & Success Criteria

Objectives:

Provide a consistent, courteous greeting experience for visitors within a controlled indoor area.

Operate safely around people and obstacles with autonomous navigation and bump recovery.

Recognize enrolled individuals locally (opt‑in, consent‑based) and customize greetings by name.

Enable rapid human handoff via a companion app with live view and push‑to‑talk.

Protect privacy by default: on‑device processing, encryption at rest, and clear consent flows.

Measurable Success Criteria (MVP Demo Readiness):

Autonomy: 60 minutes of patrol without collision; recovery from gentle bump within 1 second.

Perception: ≥95% recall for person detection in the demo environment; ≤2 false greets/hour.

Recognition (opt‑in): ≥95% top‑1 accuracy on an enrolled test set with ≥2 images/person.

Latency: detection‑to‑greet < 800 ms; handoff to human < 1.5 s from button press.

Privacy & Consent: All enrollments logged with consent; one‑click deletion verified.

3. Scope

In Scope (MVP):

Indoor differential‑drive robot with depth‑assisted obstacle avoidance and bumper failsafe.

On‑device person detection and optional face recognition (consent‑based).

BLE companion app (Flutter) for live view (low‑FPS), push‑to‑talk, greet/handoff controls, and enrollment workflow.

Local storage (SQLite) of consent records, embeddings, and visitor logs with export capability.

Basic patrol within a pre‑mapped area and a ‘greeting zone’ behavior.

Out of Scope (MVP, deferred):

Cloud‑based continuous streaming or external identity providers (e.g., SSO).

Advanced SLAM in dynamic/large spaces; multi‑floor navigation.

Emotion analysis or any sensitive‑attribute inference.

Security enforcement actions beyond greeting and notification.

Multi‑robot coordination.

4. Stakeholders & RACI (Draft)

| Role | Name/Org | Responsibility | RACI | Notes |
| --- | --- | --- | --- | --- |
| Project Sponsor | Stephen Perelgut | Vision, approvals, demo success | A | Sets priorities, signs off MVP |
| Tech Lead (Robotics) | TBD | ROS 2 stack, nav, safety | R | Leads robot bring‑up |
| Perception Lead | TBD | Detection, embeddings, thresholds | R | Accuracy & latency targets |
| App Lead (Flutter) | TBD | BLE app, UI, consent/enrollment | R | Handoff & logs |
| Privacy Officer | TBD | Policy, signage, audits | C | PIPEDA alignment |
| QA/Safety | TBD | Test plans, acceptance | C/R | Runs demo‑readiness tests |

5. User Stories (MVP)

As a visitor (unknown), I want to be greeted politely and offered help so I know how to proceed.

As an enrolled visitor (known), I want to be greeted by name so the experience feels personal.

As an operator, I want to see what the robot sees and talk through it so I can assist visitors remotely.

As an administrator, I want an easy enrollment flow that captures consent and enables deletion.

As a safety officer, I want the robot to slow/stop near people and recover safely from bumps.

6. Non‑Functional Requirements

Reliability: 1‑hour continuous operation per patrol cycle in a typical indoor setting.

Privacy: On‑device inference by default; encrypted storage for identities/logs; no cloud uploads without explicit approval.

Accessibility: Clear voice; on‑device volume control; visual indicators for status; signage with QR to policy.

Maintainability: Modular ROS 2 nodes; configuration via YAML; logs export as CSV/JSON.

Security: BLE pairing confirmation; signed app builds; local admin PIN for sensitive actions.

7. System Architecture (Overview)

Compute module (Linux + ROS 2) orchestrates navigation (Nav2), perception (person detection + optional face embeddings), greeting logic, and a BLE service. Sensors include an RGB/depth camera, microphones, speakers, and bumper switches. The Flutter app connects over BLE to show a low‑latency preview, perform handoff, and run enrollment/consent. Data is stored locally with export features for audits and deletion.

8. Hardware Requirements (Categories)

Compute: Raspberry Pi 5 + optional Edge TPU, or NVIDIA Jetson Orin Nano.

Sensors: RGB/depth camera (e.g., RealSense D435), bumper switches; optional 2D lidar for robustness.

Audio I/O: 2‑mic array; 3–5 W speaker; small class‑D amplifier.

Mobility: Differential‑drive base with encoders; motor driver; odometry feedback.

Power: Li‑ion pack with BMS; DC‑DC converters (5V/12V rails); emergency stop.

Chassis: Layered plates or 3D‑printed shell; accessible e‑stop; cable management.

9. Software Requirements (MVP)

OS & Frameworks: Ubuntu 22.04 LTS; ROS 2 Humble; OpenCV; Nav2; rclcpp/rclpy.

Perception: Person detector (e.g., YOLO/MediaPipe) with tracking; embeddings for face recognition (ArcFace/FaceNet‑style).

Speech: On‑device ASR (e.g., Whisper small/Vosk); TTS (local engine or pre‑recorded prompts).

App: Flutter (iOS/Android/Desktop) with BLE; live view (low‑FPS MJPEG/RTSP); enroll/consent; logs view/export.

Data: SQLite for identities/logs; AES‑encrypted columns or FS‑level encryption; nightly backup to encrypted volume (optional).

10. Data Governance & Privacy (Draft)

Consent‑first enrollment; prominent signage at entry points; QR code to policy and contact.

Purpose limitation: use images/embeddings only to personalize greetings; no secondary uses.

Minimal retention: store embeddings and consent record; no raw training sets retained by default.

Right to access & deletion: Admin UI supports lookup and immediate deletion; actions logged for audit.

Security controls: encryption at rest; device‑level full‑disk encryption; admin PIN; change logging.

11. Safety Requirements

Max speed limit in proximity to people; dynamic slowdown near detected humans.

Physical e‑stop accessible and clearly marked; software watchdog for runaway states.

Bump detection triggers immediate stop and re‑plan; path planning respects keep‑out zones.

Battery and power protections: BMS, fusing, proper cable strain relief; safe charging procedures.

12. Operational Scenarios (Happy Paths)

Patrol and greet unknown visitor → polite message → offer handoff → operator assists via app.

Known visitor approaches → robot recognizes → greets by name → offers assistance menu or handoff.

Enrollment: operator requests consent → capture name + 2–3 images → embeddings stored locally.

Bump event: robot stops → announces ‘excuse me’ → re‑plans path → resumes patrol.

13. MVP Delivery Plan (8–10 Weeks)

Phase 0 (Week 1): Charter, safety plan, privacy policy, acceptance tests seeded.

Phase 1 (Weeks 2–4): Platform bring‑up, teleop, person detection, generic greeting, BLE app live view.

Phase 2 (Weeks 5–7): Enrollment/consent, name greeting, unknown‑visitor flow, human handoff, basic patrol.

Phase 3 (Weeks 8–10): Hardening, logs/exports, demo scripts, signage and end‑user docs.

14. Key Risks & Mitigations (Snapshot)

False recognition: threshold + confirmation (‘Did I get your name right?’) + easy correction.

Lighting variability: use depth + auto‑exposure; small IR assist in low light.

Acoustic noise: push‑to‑talk prioritizes operator; use wake‑word only in quiet zones.

BLE reliability: keep sessions short; Wi‑Fi fallback for setup; reconnect logic.

Battery life: duty‑cycle compute; configurable patrol cadence; low‑power standby.

15. Acceptance Criteria (Go/No‑Go for MVP Demo)

All success criteria in Section 2 met in the demo environment.

Consent and deletion flow demonstrated for at least two individuals.

Patrol with at least five successful greetings (mixed known/unknown) in 15 minutes.

Emergency stop and bump recovery demonstrated visibly and reliably.

16. Assumptions & Constraints

Indoor, level flooring with reasonable lighting; defined patrol area available.

Local regulations and institutional privacy policies allow consent‑based recognition.

Hardware availability may affect component selection; alternates permitted if spec‑equivalent.

No dependency on external internet connectivity during demos.

17. Deliverables (MVP)

Operational robot prototype with patrol, greet, and handoff features.

Flutter companion app (test builds) with enrollment and logs.

Documentation pack: setup guide, operator guide, privacy policy, signage PDF.

Test reports: performance metrics, safety checks, and acceptance results.

18. Glossary (Select)

BLE: Bluetooth Low Energy for short‑range, low‑power connectivity.

Embedding: Vector representation of a face used for recognition (not a photo).

Nav2: ROS 2 Navigation stack for mapping and path planning.

PIPEDA: Personal Information Protection and Electronic Documents Act (Canada).

— End of v0.2 —

This document is a working draft intended for internal review and iteration.